Article

Machine Learning-Based Mobile Traffic Classification for 
QoS-Oriented Network Management

Mohammed Aqeel Ismail * and Okuthe P. Kogeda *

School of Agriculture and Science, College of Agriculture, Engineering and Science, University of 
KwaZulu-Natal, Pietermaritzburg Campus, Pietermaritzburg 3201, South Africa

* Correspondence: 221071308@stu.ukzn.ac.za (M.A.I.); kogedao@ukzn.ac.za (O.P.K.)

Abstract

The increasing complexity and volume of mobile network traffic present significant challenges to maintain consistent Quality of Service (QoS) across diverse applications. Accurate traffic classification enables application-aware resource allocation by distinguishing applications with different bandwidth, latency, and reliability requirements. Traditional classification techniques, including port-based identification and Deep Packet Inspection (DPI) have become inadequate and less effective due to widespread encryption, port masquerading, and growing privacy concerns. This paper presents a supervised learning-based approach for application-level network traffic classification specifically as a foundation for QoS optimization in future 5G networks. Since publicly available labeled 5G traffic datasets remain limited, this study uses the MIRAGE-2019 mobile traffic dataset as a proxy dataset to evaluate the proposed classification framework. A Random Forest classifier was implemented using flow-level statistical features extracted from the mobile application traffic. The framework further incorporates a rule-based QoS policy mapping informed by RFC 4594 DiffServ service class guidelines to assign application-specific priority levels, bandwidth requirements, latency sensitivity, and jitter tolerance. Experimental evaluation achieved an overall classification Accuracy of 71.83%, a Macro F1-score of 0.6701, and a Weighted F1-score of 0.7227 across twenty mobile applications. Although the experiments were conducted using a pre-5G mobile traffic dataset, the results demonstrate that supervised machine learning can effectively classify encrypted mobile application traffic and provide a practical foundation for application-aware QoS policy enforcement in future 5G and next-generation mobile networks.

Academic Editor(s): Paolo Bellavista Received: date Revised: date Accepted: date Published: date Copyright: © 2026 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license.

Keywords: 5G networks; network traffic classification; Quality of Service (QoS); statistical features; supervised learning; random forest algorithm; machine learning; MIRAGE2019

1. Introduction

The global dependence on internet connectivity has grown exponentially, reshaping how individuals and organizations operate, communicate and access services. This shift is largely driven by advances in networking technologies, most notably, 5G (the Fifth Generation) of wireless network technology that succeeds the 4G LTE standard [1].

5G introduces transformative capabilities through enhanced mobile broadband (eMBB), ultra-reliable low-latency communications (URLLC), and massive machine-type communications (mMTC), enabling support for a wide range of emerging applications with diverse Quality of Service (QoS) requirements [2]. 5G’s key features include the use of wider spectrum utilization, including sub-6 GHz and millimeter-wave frequency bands that the previous generation’s wireless technologies did not offer. This allows users to expect greater capacity, faster data transfer rates, and stable connections but also requires more network infrastructure to be deployed to ensure reliable coverage that supports new applications, while simultaneously increasing the complexity of network traffic management.

As the number of users and applications continues to grow, efficient allocation of network resources becomes increasingly important. This creates a challenge to ensure that each application receives the appropriate level of network resources to perform optimally. For instance, video conferencing and online gaming require low latency and high reliability, whereas services like software updates are more tolerant to delay [3]. Without an intelligent mechanism to distinguish between these traffic types, network resources may be allocated inefficiently, leading to Quality of Service being degraded. This is where the application of network traffic classification and QoS come into play.

Network traffic classification is a growing domain within computer science and network engineering. Network traffic represents the amount of data transmitted over a network over a given time, for communication and exchange of information between interconnected devices within a network. In the context of Internet Service Providers (ISPs) and mobile network operators [4] [5], the accurate identification and categorization of network traffic according to application type is an important element of many network management tasks [6], such as network monitoring, resource allocation, security, traffic engineering, and QoS enforcement.

Accurate classification allows network operators to understand network behavior, which is a critical step for network security, to detect malicious behavior, efficient resource management and enforce Quality of Service (QoS) policies tailored to applications needs [7]. In next-generation mobile networks, QoS mechanisms prioritize critical services, allocate bandwidth efficiently, and ensure that latency-sensitive applications receive the performance required to operate effectively [8].

It helps to minimize the issues of packet loss (packets lost during transmission), latency (delay in data transmission) and jitter (inconsistencies in delay from one packet to the next). But this would only be possible if the network would be able to identify what type of application is generating the traffic for the application to receive the necessary resources and performance, in essence optimizing QoS.

Historically, traffic classification relied on methods such as port-based identification and Deep Packet Inspection (DPI). While simple and effective in the past, these techniques have become obsolete because of widespread encryption, port masquerading, port randomization, and privacy regulations. As a result, they fail to accurately identify traffic types.

To address these limitations, recent research has turned to machine learning classification techniques for network traffic classification. Machine learning models can analyze flow-level statistical features, such as packet size, flow durations, inter-arrival time, and byte count, without the need to inspect the packet payload.

Previous studies have demonstrated the effectiveness of Random Forest for traffic classification. However, comparatively fewer studies have investigated how classification outputs can be translated into practical QoS policy recommendations for application-aware resource management. This study therefore extends existing work by implementing a Random Forest-based application classification framework together with a QoS policy mapping mechanism that associates classified applications with priority levels, bandwidth requirements, latency sensitivity, and jitter tolerance. The objective is not to perform live QoS enforcement, but to demonstrate how accurate application classification can serve as a foundation for future QoS-aware traffic management.

Accordingly, this study addresses the main research question: How can supervised machine learning be designed and evaluated using labeled mobile application traffic to accurately classify application-level network traffic as a foundation for future QoS-aware management in 5G networks?

To answer this question, this study implements a Random Forest-based network classification system using the MIRAGE-2019 mobile traffic dataset. Since large-scale public datasets containing labeled operational 5G application traffic remain limited, the MIRAGE-2019 dataset serves as a proxy dataset consisting of representative labeled mobile application traffic rather than native 5G traffic.

In this paper, the term proxy study refers to the evaluation of the proposed framework using a representative labeled mobile traffic dataset in place of real 5G traffic because an adequately sized public 5G dataset was unavailable. Consequently, the reported results demonstrate the effectiveness of the proposed classification framework on representative mobile traffic and should be interpreted as a foundation for future application-aware QoS management in next-generation mobile networks rather than as a direct evaluation of deployed 5G networks.

The remainder of this paper is organized as follows. In Section 2, we provide a literature review and the research gap being addressed in this paper. In Section 3, we provide the methodology. In Section 4, we discuss the results along with testing and evaluation. In Section 5, we provide the practical implications. In Section 6, we provide the conclusion and areas of consideration for future work.

2. Literature Review

Traditional traffic identification methods such as port-based detection and Deep Packet Inspection (DPI) were once effective but now struggle with modern traffic, and have been gradually replaced by machine learning approaches, including supervised, unsupervised, and semi-supervised learning.

2.1. Port-Based and DPI Methods

The earliest techniques for network traffic classification relied on port-based classification. Port-based classification identified application types based on the port numbers associated with network packets. It would check the source or destination port of the packet headers of transport layer protocols, namely, TCP (Transmission Control Protocol) and UDP (User Datagram Protocol) [9] and identify the application type based on this mapping. This approach was initially effective as the Internet Assigned Numbers Authority (IANA) [27] registered “well-known port numbers” [4] to many services. The traffic is identified by the registered port number the application is registered to [10]. For example, email applications use the 25 (SMTP) port number to send emails and to receive emails the 110 (POP3) port is used. And web applications use the 80 port number which is HTTP (Hypertext Transfer Protocol).

Port-based classification has become increasingly unreliable since modern applications often use dynamic or non-standard ports and peer-to-peer (P2P) applications often disguise their traffic by operating on common port numbers to avoid detection, such as HTTP. As a result, port-based methods are prone to both misclassification and incomplete identification [11].

For instance, researchers have evaluated that port-based classification could rarely identify 70% of application traffic. Their study noted both underestimation and overestimating of classes, highlighting that “…traffic flows can also be misidentified…”  [11]. Similar limitations are noted in a 2006 paper [12] where port-based classification proved to be ineffective in identifying 30–70% of P2P traffic, primarily due its reliance on standard ports. These limitations have led to the decline of port-based classification in modern traffic classification systems.

To address the limitations of port-based classification, Deep Packet Inspection (DPI) emerged as a more accurate alternative by inspecting payload content of packets rather than relying solely on headers or port numbers [13]. DPI systems compare packet payloads against predefined applications signatures, enabling precise identification of traffic, even in complex protocols. The DPI process involves inspecting packet data for known strings, patterns or byte sequences and matching these against a signature library [1].

Another significant advancement came with the introduction of the length-based matching (LBM) framework [28], which optimized regex pattern matching for high-speed packet processing. LBM employs a novel pattern-matching engine called StriD2FA (Stride-DFA) to reduce the portion of the data stream that needs inspection, thus enhancing both speed and memory efficiency. However, this study shows that even optimized DPI has limitations with modern encrypted traffic.

Despite being more accurate than port-based classification, it comes with several drawbacks. DPI systems are computationally intensive, requiring significant memory and processing power. Moreover, DPI becomes increasingly ineffective since it cannot inspect encrypted payloads [13] [4]. Additionally, DPI raises privacy and legal concerns, as it involves inspecting user data, which may violate data protection regulations [1].

These limitations have driven the shift toward statistical and machine learning classification methods. The methods adopt the use of statistical flow-level statistical features, which are not affected by encryption and provide a scalable alternative for traffic classification [9].

2.2. Supervised Learning

As encryption and port randomization became common, supervised machine learning emerged as a dominant method for traffic classification. It has been extensively employed in network traffic classification due to its capacity to manage complex datasets and deliver high classification accuracy. Supervised learning is a type of machine learning where the model is trained using labeled data (data where the correct application/service type is already known). From the labeled dataset the model learns a set of classification rules to predict unknown traffic [4] [9].

To show a few of its outcomes [10] conducted an extensive comparative evaluation of four machine learning algorithms on flow-level traffic datasets without payload, IP or port information. They split datasets into training and testing and measured accuracy and computational overhead. In the experiment, supervised learning algorithms such as the Support Vector Machine (SVM) and Naïve Bayes algorithms achieved Accuracy outcomes of 74,05% and 71,89%, respectively [10].

A recent study [5] found that algorithms like Random Forest, Naïve Bayes and K-Nearest Neighbors can be used for classification, with each achieving a classification Accuracy of more than 0.90 after implementing various other machine learning techniques like SMOTE. Their results claim that machine learning classifiers improved QoS optimization and bandwidth management. Their study is very closely related to what this paper has accomplished.

Ganesan et al [7] in their study addressed the challenges of managing and classifying network traffic in smart IoT environments. The authors used supervised learning models in statistical features extracted from traffic flows, testing performance across IoT-specific datasets. They found that Random Forest outperformed other models in terms of Accuracy, Precision, and execution time, making it ideal for smart and time-sensitive networks [7].

Azab et al [1] present a comprehensive survey of network traffic classification approaches, discussing traditional techniques as well as machine learning methods. Their work provides a foundational understanding of classification methods. They conclude that machine learning, particularly supervised machine learning is built using statistical flow-level features; it is relatively easy to implement, offers high accuracy, uses minimal computational power, detects small grains, and does not require access to payload contents, making it suitable for modern encrypted and heterogeneous traffic [1]. It does, however, have drawbacks. The examples used in the training dataset have a significant impact on the classifier’s performance to recognize applications. For instance, if there is a new application the model will not be able to identify it. For application to be classified the model will have to be retrained with the new application.

2.3. Unsupervised Learning

Unsupervised learning is a useful complement to supervised learning especially when using unlabeled datasets. Unsupervised learning methods include the use of clustering algorithms to identify natural grouping in traffic. The goal is finding patterns, similarities, or groups in the data without being told what the data represents [1].

Erman et al [6] demonstrated the use of K-means clustering to separate flows into application-based clusters using transport layer statistical features. They concluded that unsupervised learning could achieve meaningful application traffic separation without the need for payload inspection.

Canever and Wang [3] in their study evaluate various unsupervised clustering algorithms for grouping network traffic flows based solely on statistical flow features, addressing scenarios where labeled data is unavailable. The authors selected clustering algorithms, namely K-means, which is known for its simplicity, and aims to classify data by dividing it into K clusters. They showed that unsupervised methods can detect unknown traffic types but often lack labeling accuracy, making it less accurate than supervised models [3].

These techniques are particularly valuable when dealing with unknown or encrypted traffic types. However, unsupervised methods alone do not assign meaningful class labels, which limits their use in QoS enforcement unless paired with manual or rule-based interpretation. In a QoS context, one can assign policies per cluster, but human interpretation is still needed to map clusters to application types. Nevertheless, unsupervised clustering lays the groundwork for identifying traffic structure when labels are unavailable.

2.4. Semi-supervised Learning

Semisupervised learning combines the strengths of supervised and unsupervised techniques. This is especially relevant in 5G contexts, where new traffic types emerge regularly and labeling all data is impractical. It starts with unlabeled traffic and cluster it to generate useful structure or labels. Then, you train a supervised model on that processed data to classify future traffic flows.

Labayen et al [14] proposed a new three-layer model to classify captured traffic data by analyzing user behavior. The authors used a hybrid approach of unsupervised K-means clustering algorithm and supervised Random Forest to achieve real-time classification across common user activities (web browsing, video streaming, VoIP). Even though the model achieved a mean Accuracy of 96,09%, their experiment shows that their approach has a significant impact on computational speed and memory usage, which led to longer training and testing times.

Nguyen and Armitage [9] proposed a semisupervised network traffic classification system that combined K-means clustering and Naive Bayes classification using flow-level statistical features. Their system required only a small percentage of labeled data (as little as 10%) to accurately classify traffic flows. The study in [15] highlighted that semisupervised learning could reduce the dependency on data-labeled datasets while still achieving high accuracy, outperforming traditional port-based classification methods.

Even though it provides high accuracy it requires higher computational resources than supervised and unsupervised solutions since it combines both unsupervised and supervised learning.

These findings consistently show that machine learning-based traffic classification, especially supervised learning and the use of statistical flow features, provide a practical and effective alternative to traditional methods. The only drawback of supervised machine learning is obtaining a labeled dataset.

The drawback of a labeled dataset is solved by the open dataset, the MIRAGE-2019 dataset. The insights from the literature directly form the methodology for this project, which encourages the implementation of a supervised learning using a Random Forest-based classifier to optimize QoS in the MIRAGE-2019 dataset.

2.5. Research Gap

Research shows the effectiveness of machine learning in traffic classification but does not use the classifier outputs for actionable QoS policy implementations. Most of the literature prioritizes classification but does not offer practical QoS policy applications. Here, the contribution of the study lies within the gap in the literature by extending flow-based statistical feature traffic classification with policy mapping that offers actionable QoS recommendations at the classifier output level.

3. Methodology

3.1. Dataset

A labeled dataset is crucial for supervised machine learning because it provides the model with labeled features that will guide the learning process and enable performance evaluation [1].

Aceto et al [16] created the MIRAGE-2019 dataset used in this experiment. It is important to note that MIRAGE-2019 is not a native 5G network dataset. The traffic was collected between May 2017 and May 2019, prior to the widespread commercial deployment of 5G networks, and therefore does not contain 5G-specific characteristics such as network slicing, ultra-reliable low-latency communications (URLLC), massive machine-type communications (mMTC), and enhanced mobile broadband (eMBB). Consequently, this study should be interpreted as a proxy study that uses labeled mobile application traffic to evaluate an application-level traffic classification framework rather than a direct evaluation of live 5G network traffic. The findings therefore demonstrate the effectiveness of the proposed classification approach on representative mobile traffic and provide an initial step toward future QoS-aware traffic management in 5G networks.

Despite this limitation, MIRAGE-2019 remains one of the most comprehensive publicly available labeled mobile application traffic datasets. The MIRAGE-2019 dataset is a human-generated dataset that was produced for mobile traffic analysis. It contains labeled bidirectional traffic flows generated by over 280 experiments using 40 mobile apps across three devices, viz., Google Nexus 7, Xiaomi Mi5 and Samsung Galaxy A5. Over 280 students (ages 19–25) from three courses at the University of Napoli “Frederico II” voluntarily participated. Each student conducted one or two experimental sessions. The participants performed typical app usage activities, including exploring functionalities, first-time installation, login, and registration processes, to generate realistic traffic data. The dataset is made available in JSON format making it compatible and user friendly. The dataset gathers traffic generated by 40 android apps belonging to 16 different categories according to the Google Play app distribution platform.

This study is based on only a sample of the dataset made available on Kaggle. It consists of data from 20 applications generated by Google Nexus and Xiaomi Mi5. The mobile app traffic was collected for a period of 2 years (May 2017–May 2019) and openly released to the research community. This was done to ensure transparency and replicability in traffic analysis and to study traffic patterns as well as to examine models and perform classification tasks [16].

The dataset was selected because it provides accurately labeled application-level network flows required for supervised learning and offers sufficient data volume to train and evaluate machine learning classifiers. Although the traffic does not originate from a 5G network, many of the extracted flow-level statistical features, including packet sizes, flow duration, packet counts, byte counts, and inter-arrival times, are protocol-independent traffic descriptors that remain applicable to application-level traffic classification irrespective of the underlying mobile generation. Accordingly, this study evaluates the proposed machine learning framework using representative mobile traffic as a proxy, while recognizing that additional validation on real 5G datasets is necessary before conclusions can be extended to operational 5G network deployments.

The original intention of this research was to use the publicly available 5G traffic dataset presented by Choi et al. [17]. However, after pre-processing and bidirectional flow aggregation, the dataset contained an insufficient number of labeled flows to train and evaluate a supervised machine learning classifier reliably. The limited number of training samples significantly reduced the model’s ability to generalize across application classes. Consequently, MIRAGE-2019 was selected as an alternative because it provides a substantially larger volume of labeled application traffic and has been widely used for benchmarking application-level traffic classification models.

The MIRAGE-2019 dataset includes traffic from diverse application categories as shown in Table 1:

Table 1. Categories of applications in MIRAGE-2019 dataset.

| No. | Application | Category |
| --- | --- | --- |
| - 1 | Waze | Navigation |
| - 2 | Viber | VoIP(Voice over Internet Protocol)/Message |
| - 3 | Messenger | VoIP/Message |
| - 4 | Slither.io | Gaming |
| - 5 | Spotify | Music Streaming |
| - 6 | YouTube | Video Streaming |
| - 7 | Facebook | Social Media |
| - 8 | Twitter | Social Media |
| - 9 | Pinterest | Social Media |
| - 10 | Wish | E-Commerce |
| - 11 | Subito | E- Commerce |
| - 12 | Groupon | E-Commerce |
| - 13 | Accuweather | Weather |
| - 14 | TripAdvisor | Travel |
| - 15 | FourSquare | Travel |
| - 16 | Iliga | Sports |
| - 17 | Dropbox | Cloud Storage |
| - 18 | Trello | Productivity |
| - 19 | Duolingo | Productivity |
| - 20 | Comics | Entertainment |

3.2. Categories of Features in MIRAGE-2019

Each flow in the MIRAGE-2019 dataset is represented as a collection of statistical features derived from bidirectional packet streams. The dataset organizes these into feature categories, each describing different aspects of flow behavior. For each category, MIRAGE-2019 contains statistical summaries such as minimum, maximum, mean, standard deviation and skewness.

These features capture protocol-level characteristics of network flows, allowing the model to learn fine-grained behavioral signatures unique to each application.

Packets: This is the number of packets per flow in the forward and backward directions. These statistical features (min, max, std, skewness) are calculated for each direction separately.

Bytes: This is the total bytes in a flow. Statistical features are calculated per direction.

Inter-Arrival Time (IAT): IAT measures the time gaps between consecutive packets in a flow. It includes statistical features, min, max, mean, std, skewness, for each direction.

Flow Duration: This is the total duration from the first to the last packet in the flow, generally treated as a single feature. It is not split by directions. Example: flow_duration.

Active and Idle Times: These measure periods of activity (packets being transmitted) and inactivity within the flow, with statistical features (min, max, mean, std, skewness). Example: active_forward_max.

Flags: These are the counts of TCP (Transmission Control Protocol) flags (e.g., PSH, URG, FIN, ACK, SYN, RST) observed in the flow, typically in the forward or backward direction depending on the implementation. Example: forward_psh_flags.

Throughput or Rates: These are measures of the traffic rate, expressed in packets/sec or bytes/sec, computed separately for the forward and backward directions. Example: bytes_per_second_backward.

The following statistics are calculated by MIRAGE-2019 for most feature categories; it includes min which is the minimum value, max which is the maximum value, mean which is the average, std the standard deviation, skewness–asymmetry in the feature distribution and sum which is sometimes included for total aggregation.

Thus, combinations of direction (forward/backward) and statistic type (min, mean, std, etc.) might result in numerous derived features (e.g., packet_size_forward_mean, packet_size_backward_std) from a single logical feature, such as packet size.

Because they capture the unique behavioral fingerprints of every application, these statistical flow-level attributes are essential for application-level traffic classification. The Random Forest classifier can efficiently learn to differentiate between different application types and deduce their QoS attributes, including latency sensitivity and bandwidth requirements, by analyzing these distributions.

3.3. Ethical Considerations Related to the Dataset

Since participants were alerted of the goals and potential public release of traces. The capture procedure and dataset collecting do not raise any ethical issues. Logged source IP addresses are part of the private IPv4 space, and mobile devices were supplied and utilized in the ARCLAB. From the capture stage until the construction of the dataset and throughout the classification procedure, no participant personal information was used [16]. To protect privacy, this project only uses publicly accessible datasets. In accordance with ethical guidelines for network data study, no personally identifiable information is gathered or processed.

3.4. MIRAGE-2019 Dataset Justification and 5G Limitations

There are currently few publicly accessible datasets that depict actual 5G network traffic with identified application classes. Existing 5G datasets are frequently simulation-based, privately owned, require paid access, or contain insufficient labeled application flows for training supervised machine learning models.

Consequently, this study employs the publicly available MIRAGE-2019 dataset, despite it pre-dating widespread commercial 5G deployment. MIRAGE-2019 should therefore be regarded as a representative mobile application traffic dataset rather than a native 5G traffic dataset.

This restriction draws attention to a crucial area of research: the requirement for large-scale, publicly accessible, labeled 5G traffic datasets that enable rigorous development and evaluations of machine learning models for QoS-aware traffic management in operational 5G networks.

A publicly available 5G traffic dataset [17] was initially considered for this research. However, following pre-processing and bidirectional flow aggregation, the dataset contained an insufficient number of labeled flows to train and evaluate a supervised machine learning classifier reliably.

The MIRAGE-2019 dataset was selected because it provides a substantially larger and more diverse collection of labeled application flows, making it more suitable for training the supervised machine learning models used in this study.

3.5. Implementation Environment

The network traffic classification model was developed in Python 3.13.6 using the Visual Studio Code environment. It relies on many open-source computing and machine learning libraries and packages, summarized in Table 2 (“Joblib: running Python functions as pipeline jobs—joblib 1.5.2 documentation,”) [18].

Table 2. Python dependencies.

| No | Package | Purpose |
| --- | --- | --- |
| 1 | Pandas 2.3.1 | Data manipulation and analysis |
| 2 | Numpy 2.3.2 | Numerical computing |
| 3 | Scikit-learn 1.7.1 | Machine learning algorithms and evaluation metrics |
| 4 | Matplotlib 3.10.5 | Visualization and plotting |
| 5 | Seaborn 0.13.2 | Statistical data visualization |
| 6 | Imbalanced-learn 0.14.0 | Oversampling and data balancing techniques |
| 7 | Joblib 1.5.1 | Model persistence and serialization |
| 8 | Scipy 1.16.1 | Scientific and mathematical operations |

3.6. Flow Aggregation

The MIRAGE-2019 dataset consists of flow-level JSON files, each describing statistical characteristics of bidirectional network communications produced by a particular mobile application. Although the data is derived from raw packet captures, the MIRAGE dataset already provides flow-level statistical features. This simplifies the pre-processing stage by removing the need for flow computation.

In the proposal it was mentioned that flow computation might be necessary if the dataset contains raw PCAP files, where it would require computing and extracting the flow-level statistical features, but the MIRAGE-2019 dataset contains JSON files with flow-level statistical features.

Each flow has the key “192.168.20.101,51221,216.58.205.42,443,6” which is represented as follows, 192.168.20.101 is the “Source IP”, 51221 is the “Source Port”, 216.58.205.42 is the “Destination IP” (Google’s domain here, Twitter), 443 is the “Destination Port” (HTTPS), and 6 is the “Protocol Number”, where in this case 6 is TCP.

The string “192.168.20.101,51221,216.58.205.42,443,6” functions as a flow ID or 5-tuple in the MIRAGE dataset. It uniquely identifies a bidirectional flow by specifying the source IP and port, destination IP and port, and the protocol number. This 5-tuple serves as a key to associate all related data (per-packet info, flow features, metadata) within the dataset.

Every JSON file has a set of flows, each of which includes a variety of network metrics arranged in a layered hierarchical structure, including packet lengths, inter-arrival periods, and TCP window data.

But models like the Random Forest classifier and machine learning frameworks like Scikit-learn need tabular, structured numerical input. The data had to be converted from JSON to CSV format because the MIRAGE JSON files are hierarchical and unstructured for direct ML input. In this structure, a single network flow is represented by each row, a numerical feature is represented by each column, and the originating application is specified by a single label column.

To prepare the dataset for supervised learning, a python script was developed. This script performs the flow aggregation process using two primary methods: process_file() and aggregate().

The aggregate() function iterates through all JSON files, calling process_file() for each file, and then concatenate the individual flow data frames into a single dataset.

Each JSON file is read by the process_file() function, which then extracts relevant flow-level statistical features, including the TCP window attribute, packet length statistics, and inter-arrival time metrics, and arranges them into a flat data frame. The function gathers and flattens the hierarchical JSON structure’s flow-level statistical features into a format that is suited for machine learning.

Flow-level statistical features get saved in the dataframe as the following features, feat_cat which is the category of feature (e.g., packet_size, inter-arrival time), direction which tells flow direction (either forward or backward), stat_name which is statistical measure (mean, std, min, max), val which is the statistic’s numerical value, and col_name which makes each extracted statistic a column in the dataframe by creating a unique column name such as packet_size_forward_mean.

By merging the JSON data from the Google Nexus and Xiaomi Mi5 smartphones into a single input directory, the model was able to learn from a greater variety of flow patterns across different hardware and software scenarios. The combined dataset produced more realistic and widely applicable results.

Finally, the exported CSV file Mirage_flows.csv represents the complete set of flow-based features and related applications labels. The Random Forest model is utilized for feature scaling, resampling, training, and classification in the supervised learning pipeline, which receives the Mirage_flow.csv file as input.

3.7. Data Pre-Processing

After combining all flow-level statistical variables into a single CSV file, Mirage_flow.csv, the data undergoes a pre-processing procedure to ensure compatibility with the machine learning model and enhance classification performance. This stage guarantees that the dataset is suitable for supervised machine learning in terms of cleanliness, balance, and structure.

Each network flow in the Mirage_flow.csv dataset has a single row with hundreds of statistical variables that describe the characteristics of bidirectional communication. The column label identifies the original mobile application (like Facebook, YouTube, or Twitter).

During pre-processing, the dataset is divided into:

Feature matric (X): This contains numerical flow-level features describing bidirectional flow characteristics such as packet sizes, inter-arrival times and TCP statistics;

Label vector (y): This contains the corresponding mobile application name for each flow (e.g., YouTube, Twitter, Spotify), the labels.

This division ensures that the classification algorithm is trained on the input features (X) to predict the correct output label (y).

3.8. Handling Missing and Constant Values

A few flows in the MIRAGE dataset had missing or invalid entries due to incomplete network sessions. The pre-processing function replaces all missing values (NaN) with zero using X.fillna(0).

This approach avoids discarding incomplete flows, preserving dataset size while maintaining numerical compatibility for the classifier.

To ensure the model does not learn noise or redundant patterns, missing values are replaced with zero (0) using the fillna() method.

Features with zero variance (i.e., identical values for all flows) are removed to improve efficiency and prevent overfitting.

This step ensures that only statistically meaningful features contribute to model training. As such, features provide no discriminative power for classification and only increase computational cost.

3.9. Dataset and Application

Before transformation, the pre-processing function provides a summary of dataset characteristics including:

The total number of features and flows: Dataset shape: 121,955 flows, 102 features each;

The total number of unique applications: Number of apps: 20;

The number of flows per application (see in Table 3).

This information is crucial for understanding the dataset’s structure and identify class imbalance, where certain applications may dominate in number of flows.

Table 3. Number of flows per app.

| No | Identity | Number of Flows |
| --- | --- | --- |
| 1 | Waze | 11,865 |
| 2 | Iliga | 11,048 |
| 3 | accuweather | 10,682 |
| 4 | duolingo | 8583 |
| 5 | subito | 8275 |
| 6 | Wish | 6812 |
| 7 | spotify | 6615 |
| 8 | foursquare | 6549 |
| 9 | youtube | 6493 |
| 10 | twitter | 5767 |
| 11 | comics | 5662 |
| 12 | facebook | 5578 |
| 13 | dropbox | 5205 |
| 14 | pinterest | 4253 |
| 15 | messenger | 4133 |
| 16 | tripadvisor | 3676 |
| 17 | slither | 3189 |
| 18 | viber | 3118 |
| 19 | trello | 2380 |
| 20 | Groupon | 2072 |

A few flows in the MIRAGE dataset had missing or invalid entries due to incomplete network sessions. The pre-processing function replaces all missing values (NaN) with zero using X.fillna(0). The bar chart (see in Figure 1) titled “Application Traffic flows” presents the distribution of traffic flows across different applications. Each bar represents the number of flows associated with a specific application. The color coding corresponds to the priority classification under a QoS (Quality of Service) policy framework: High Priority (dark cyan), Medium Priority (pale orange), and Low Priority (light peach)

Figure 1. This bar chart illustrates the number of flows per app.

The number of flows is displayed on the y-axis, and apps are listed on the x-axis in descending order of traffic volume.

Key observations include:

- Highest Traffic Application:

  - Waze dominates with nearly 12,000 flows, indicating significant usage of navigation services;

  - Accuweather, Duolingo and Iliga also show high traffic volumes (8000–11,000 flows);

  - This suggests that navigation, weather, sport and language learning apps are among the most frequently used categories in the dataset.

- High-Priority Apps

Waze, Spotify, and Viber are classified as High Priority, due to their real-time needs:

  - Waze: Requires low latency for real-time navigation;

  - Spotify: Needs stability for continuous listening;

  - Viber: Demands high reliability for voice over IP.

- Medium-Priority Apps

  - Majority of applications (e.g., Accuweather, Subito, YouTube, Twitter, Facebook) fall into Medium Priority;

  - These applications need a reasonable amount of bandwidth but can withstand moderate latency;

  - Applications like YouTube and Facebook have significant flow counts, reinforcing their popularity in general usage.

- Low-Priority Apps

  - Duolingo, Dropbox, Trello, and Comics are mapped as Low Priority;

  - These apps generally involve background synchronization or non-real-time activity, making them less sensitive to latency;

  - Their flow counts are lower compared to High- and Medium-Priority apps, aligning with their reduced urgency.

From the bar chart it can be noticed that the traffic is unevenly distributed, with a few apps accounting for most flows (Waze, Accuweather, Motain, Duolingo). The bar chart assists in understanding the contribution of each application to the dataset and guides the choice of balancing techniques.

3.10. Model Training

Once the model was pre-processed and structured, the next stage involved training the supervised learning model. The dataset was partitioned into training and testing subsets using an 80:20 stratified train–test split. Stratification preserved the relative distribution of application classes across both subsets. The test set remained completely unseen during model development and was reserved exclusively for final performance evaluation. The train_model() function in this study performs three critical steps:

Feature scaling (“StandardScaler,”) [29];

Class balancing through oversampling [5]; and

Model training using Random Forest [19].

Additional, Linear Discriminant Analysis (LDA) can optionally be applied for dimensionality reduction before model fitting [5].

All subsequent pre-processing operations, including feature scaling, oversampling and optional dimensionality reduction, were performed only on the training data. This prevents information leakage from the test set into the training process and provides an unbiased estimate of model generalization performance.

3.11. Feature Scaling

The dataset contains numerical features measured on different scales. Packet length has values like ~52.0, ~1200 (in bytes). Inter-arrival time might be in milliseconds (with values like ~0.05, ~5445.071). The model could become biased towards features with larger numeric ranges. If the model is given these raw numbers directly, the features with larger numeric values dominate the learning process, while features with smaller values will get ignored even if those smaller values are just as important. To prevent features with larger numeric ranges from dominating the learning process, standardization was performed using the StandardScaler from Scikit-learn (“scikit-learn: machine learning in Python—scikit-learn 1.7.1 documentation”) [20].

The scaler (StandardScaler) solves this by putting all features on the same scale. All input features are rescaled to a standardized distribution with a mean of zero and unit variance, ensuring that all features contribute equally during model training.

Mathematically, each feature x is transformed as given by Equation (1):

| x=(x -u)s | (1) |
| --- | --- |

where u is the mean of the training sample and s is the standard deviation of the training samples.

The StandardScaler was fitted exclusively on the training dataset. The fitted scaler was subsequently applied to transform both the training and testing datasets using the same scaling parameters. This ensures that no statistical information from the test set influences the training process.

The fitted scaler object is serialized and stored (feature_scaler.pkl) to guarantee consistent normalization for future inferences and model deployment.

Why save the Scaler?

The scaler ensures future classification of new network flows are scaled in the same way as the training of the model. This prevents the risk of: scaling new data differently; shifting the feature distribution; and getting wrong classification during inference, when new data comes in, and at deployment. The key is to use the same scaler object the model was trained with, so the new flows are on the same scale. This guarantees consistency between training and inference, preventing scaling mismatches.

3.12. Handling Class Imbalance Using Oversampling

The MIRAGE-2019 dataset contains network traffic from 20 mobile applications, each with varying traffic volumes. Applications such as Waze, Iliga and Accuweather have significantly higher number of flows compared to lightweight apps such as Viber or Trello.

This results in a class imbalance problem (see Figure 2), where the classifier is exposed to more examples from dominant classes. Training a model on such imbalanced data can cause the model to bias toward majority classes. This reduces the model’s ability to generalize to minority classes and degrades performance metrics such as Recall and F1-score for applications with fewer flows.

To address this challenge of imbalance, two oversampling techniques were implemented within the model:

RandomOverSampler: Randomly duplicates samples from minority classes until class frequencies are balanced. This is a simple yet effective approach that ensures balanced class distribution but may risk overfitting due to repeated samples.

SMOTE (Synthetic Minority Oversampling Technique): Unlike random duplication, SMOTE synthesizes new minority-class samples by interpolating between existing samples in the feature space. This improves the model’s performance on minority classes and helps it generalize more effectively by producing more realistic synthetic instances [5].

Figure 2. MIRAGE-2019 dataset’s Confusion matrix.

Both oversampling techniques were designed as configurable toggles within the training model.

The script ensures that only one method is applied at a time. If both are enabled,

RandomOverSampling is selected by default. As a result, each application class contained 9492 flows ensuring balanced distribution across classes. In this study, RandomOversampling produced better results for the MIRAGE-2019 dataset compared to SMOTE, which is discussed later in the paper.

The oversampling techniques ensure that the classifier receives equal representation from all application categories. The selected oversampling technique was applied only to the training samples, while the testing dataset retained its original class distribution. This approach prevents synthetic or duplicated samples from appearing in both the training and testing datasets, thereby eliminating data leakage and providing a more reliable estimate of classification performance. The balancing process significantly improved class representation and increased the diversity of training samples across all applications [21].

3.13. Dimensionality Reduction (LDA)

From the dataset statistics, the MIRAGE-2019 dataset contains a total of 121,955 flows and 102 flow-level statistical features each, including metrics such as packet length, maximum flow duration, and inter-arrival time variance (IAT var). Although having such a rich feature set provides comprehensive flow characterization, it also introduces redundancy and features may overlap information. Training a model with redundant features increases computational cost, slows convergence, and may lead to overfitting, reducing the model’s ability to generalize to new data.

To mitigate these issues, the model includes an optional Linear Discriminant Analysis (LDA) stage for dimensionality reduction [5]. To ensure that the new feature set retains the most discriminative information for differentiating between applications, LDA projects the data into a lower-dimensional space that maximizes class separability.

LDA was used in this study to simplify the dataset while preserving crucial discriminative features by reducing the feature space from 102 features each to 19 features each. This reduction leads to faster model training, improved generalization, and reduced memory overhead. When enabled, Linear Discriminant Analysis (LDA) was fitted using the balanced training dataset and subsequently used to transform both the training and testing datasets before classifier training.

LDA’s effect on classification performance was assessed by testing it with both the SMOTE and RandomOverSampler oversampling techniques. According to Mohammed and Ghani [5], utilizing LDA throughout the processing step aids in eliminating noise and irrelevant variability, concentrating learning on the most informative features and improving the accuracy and resilience of the model.

3.14. Model Training Using Random Forest Classifier

After feature pre-processing, scaling and optional dimensional reduction, the dataset was used to train a Random Forest classifier. Random Forest is an ensemble-based supervised learning algorithm. The algorithm constructs multiple decision trees during training and outputs their predictions for classification [22]. This method is highly effective for network traffic classification due to its robustness against overfitting, ability to represent non-linear feature interactions, and ability to interpret through feature importance metrics.

The dataset was split into 80% training and 20% testing subsets for supervised model learning and evaluation using stratified sampling. The training subset underwent feature scaling, optional oversampling, and optional dimensionality reduction before being used to train the Random Forest classifier. The testing subset remained untouched throughout model development and was used exclusively for final evaluation. This split was stratified, meaning that the relative class distribution was preserved between the training and testing sets. Stratification helps maintain representative samples of all application types, preventing bias toward dominant classes.

80% training set:

        - Without oversampling: 97,564 flows with 102 features each;

        - With oversampling: 189,840 with 102 features each;

        - With LDA applied: 189,840 with 19 reduced features each.

- 20% testing set:

24,391 flows with 102 features each.

This split ensures that the model is trained on a sufficiently large and balanced dataset while the unseen test set provides an unbiased evaluation of its generalization performance. The LDA-based dimensionality reduction (from 102 to 19 features) further helps to minimize noise, reduce computational complexity, and enhance the discriminative capabilities of the classifier.

In this implementation, the Random Forest classifier was trained using the Scikit-learn library with the following optimized hyperparameters shown in Table 4.

Time-stamping functions were used to measure the entire training time, which gave information about the computational efficiency across configurations (e.g., with/without oversampling or LDA.

The trained model was serialized and stored as model.pkl using the Joblib package, allowing for future reuse without retraining. Similarly, the feature scaler and LDA transformer (if used) were also saved to disk to maintain pre-processing consistency during model deployment or evaluation.

All experiments were performed using random_state = 42 to ensure reproducibility. Since the objective of this study was to compare pre-processing strategies under identical experimental conditions, repeated random initializations were not performed. Future work will evaluate statistical variability through repeated experiments using multiple random seeds.

Random Forest was selected because of its better generalization skills on flow-level features and track record of effectiveness in related traffic classification studies [9][16]. The model’s ensemble structure allows for interpretability through feature importance analysis and resistance against overfitting.

Table 4. Random Forest configurations.

| Parameter | Value | Description |
| --- | --- | --- |
| n_estimators | 300 | Number of the forest’s individual decision trees. A higher value balances accuracy and speed by reducing variance and improving stability while increasing computing time. |
| max_depth | 25 | Limits tree depth to prevent overfitting while maintaining model expressiveness. |
| min_samples_split | 5 | To ensure balanced growth, a minimum number of samples is needed to split an internal node. Trees with higher values are less susceptible to slight variations in the data. 5 requires somewhat larger splits, which reduces overfitting. |
| min_samples_leaf | 2 | To avoid excessively fine partitions, a leaf node must have a minimum number of samples. Ensures leaves have enough samples to represent a group. 2 prevents very small leaves that may represent noise. |
| class_weight | “balanced” | To address any remaining imbalance, weights are automatically adjusted in an inverse proportion to class frequencies. Ensures minority classes are not ignored. |
| random_state | 42 | Ensures the model produces the same splits and results each time you run it. |
| n_jobs | −1 | Utilizes all available CPU cores for faster parallelized training. −1 tells Scikit learn to use maximum parallelism. |

4. Results and Discussion

4.1. Model Testing

The model’s performance was evaluated using both traditional classification metrics and a QOS-aware evaluation framework. The evaluation aimed to determine not only the model’s ability to correctly classify mobile application traffic, but also its suitability for supporting application-aware Quality of Service (QoS) policy assignment.

Evaluation metrics such as Accuracy, Precision, Recall, and F1-score, were used to assess the model’s performance [1] [23] [9]. Since the MIRAGE-2019 dataset contains an uneven distribution of application flows, both Macro F1-score and Weighted F1-score were reported. The Macro F1-score evaluates classification performance equally across all application classes, while the Weighted F1-score accounts for the class distribution.

The model was evaluated on its results before and after applying techniques like cross validation, SMOTE, RandomOverSampling and LDA. Time and Accuracy were monitored during testing.

4.2. Model Performance on RandomOverSampling

The Random Forest classifier achieved strong performance after applying RandomOverSampling to handle the imbalance in the MIRAGE-2019 dataset. After resampling, each application class contained an equal number of flows of 9492. The model was then trained on 80% of the data and tested on the remaining 20%.

The model achieved an overall classification Accuracy of 71.83% with a Macro F1-score of 0.6701 and a Weighted F1-score equal to 0.7227. These results demonstrate that the model can successfully distinguish between a wide range of mobile application traffic despite the inherent imbalance and similarity between several application classes.

Table 5 shows the overall model performance after using RandomOverSampling. Equations of evaluation metrics are given below [23] [9]:

Table 5. Overall model performance after RandomOverSampling.

| Metric | Value |
| --- | --- |
| Accuracy | 0.7183 |
| Macro F1-score | 0.6701 |
| Weighted F1-Score | 0.7227 |

Accuracy is given in Equation (2):

| Accuracy=(TP+TN)(TP+TN+FP+FN) | (2) |
| --- | --- |

Precision is given in Equation (3):

| Precision=TP(TP+FP) | (3) |
| --- | --- |

Recall is given in Equation (4):

| Recall=TP(TP+FN) | (4) |
| --- | --- |

F1-Score is given in Equation (5):

| F1-Score=2×(Precision×Recall)(Precision+Recall) | (5) |
| --- | --- |

Macro Precision is given in Equation (6):

| MacroPrecision=1Ci=0CPrecisioni | (6) |
| --- | --- |

Macro Recall is given in Equation (7):

| MacroRecall=1Ci=0CRecalli | (7) |
| --- | --- |

Macro F1 is given in Equation (8):

| MacroF1=1Ci=0CF1i | (8) |
| --- | --- |

Weighted F1 is given in Equation (9):

| Weighted=1Ci=0CF1i | (9) |
| --- | --- |

Micro F1 can be defined in Equation (12):

| MicroPrecision=TPTP+FP | (10) |
| --- | --- |
| MicroRecall=TPTP+FN | (11) |
| MicroPrecision=2 × MicroPrecision × MicroRecallMicroPrecision+MicroRecall | (12) |

For single-label multiclass classification, the Micro F1-score is mathematically equivalent to the overall classification Accuracy because each sample contributes to exactly one prediction.

Although Micro F1 is defined for completeness, this study primarily reports Macro F1 and Weighted F1 because the MIRAGE-2019 dataset is class-imbalanced. Macro F1 evaluates performance equally across all application classes, whereas Weighted F1 accounts for class frequencies. For single-label multiclass classification, Micro F1 is equivalent to overall Accuracy and therefore provides limited additional insight.

Table 6 is the classification report across all 20 applications. Performance varied slightly depending on the type and nature of the traffic’s application but remained consistently high across classes.

The balanced class distribution allowed the model to learn equally from all application types, improving its generalization and fairness across traffic categories. Key observations:

The highest classification performance was achieved for Waze (F1 = 0.9355), indicating that its traffic exhibits highly distinctive flow-level characteristics. Other applications such as AccuWeather (0.8212), Spotify (0.8147) and Comics (0.7991) also achieved strong classification performance.

Applications including Groupon (0.4575), Trello (0.4640), TripAdvisor (0.4741) and Viber (0.4796) produced comparatively lower F1-scores. These applications generated fewer training samples and exhibit statistical flow characteristics that overlap with other application categories, making them more difficult to distinguish.

The difference between the Macro F1-score (0.6701) and the Weighted F1-score (0.7227) indicates that the classifier performs better on applications with larger numbers of flows while maintaining acceptable performance on minority applications.

Overall, the results demonstrate that RandomOverSampler effectively reduced the influence of class imbalance during training while providing a realistic assessment of model performance on previously unseen traffic.

Table 6. Classification report for each application after RandomOverSampling.

| Application | Precision | Recall | F1-Score | Support (Flows) |
| --- | --- | --- | --- | --- |
| AccuWeather | 0.8585 | 0.7870 | 0.8212 | 2136 |
| Comics | 0.7885 | 0.8101 | 0.7991 | 1132 |
| Dropbox | 0.6125 | 0.6042 | 0.6083 | 1041 |
| Duolingo | 0.7528 | 0.7857 | 0.7689 | 1717 |
| Facebook | 0.6034 | 0.6299 | 0.6164 | 1116 |
| Foursquare | 0.7154 | 0.7176 | 0.7165 | 1310 |
| Groupon | 0.3937 | 0.5459 | 0.4575 | 414 |
| Iliga | 0.7826 | 0.7312 | 0.7560 | 2210 |
| Messenger | 0.6247 | 0.6429 | 0.6337 | 826 |
| Pinterest | 0.5880 | 0.5182 | 0.5509 | 851 |
| Slither | 0.5690 | 0.7367 | 0.6421 | 638 |
| Spotify | 0.8535 | 0.7793 | 0.8147 | 1323 |
| Subito | 0.8171 | 0.7450 | 0.7794 | 1655 |
| Trello | 0.3771 | 0.6029 | 0.4640 | 476 |
| TripAdvisor | 0.4682 | 0.4803 | 0.4741 | 735 |
| Twitter | 0.5441 | 0.5889 | 0.5656 | 1153 |
| Viber | 0.4609 | 0.5000 | 0.4796 | 624 |
| Waze | 0.9556 | 0.9161 | 0.9355 | 2373 |
| Wish | 0.7805 | 0.6894 | 0.7322 | 1362 |
| YouTube | 0.7940 | 0.7775 | 0.7857 | 1299 |
| Overall Accuracy | 0.7183 | 0.7183 | 0.7183 | - |
| Macro Average | 0.6670 | 0.6794 | 0.6701 | 24,391 |
| Weighted Average | 0.7306 | 0.7183 | 0.7227 | 24,391 |

4.3. Confusion Matrix

Figure 2 represents the confusion matrix heatmap for MIRAGE-2019 dataset.

First Y-axis is the actual application labels from the dataset;

X-axis is the predicted application labels produced by the model;

Diagonal cells (top left to bottom right) are the correct classifications, where the actual application match the predicted app;

Color intensity indicates the number of flows, red shades indicate higher counts, while blue indicates lower counts.

Key observations:

First, a clear diagonal pattern is visible, indicating that most application flows were correctly classified. This demonstrates that the Random Forest classifier successfully learned discriminative flow-level statistical features for many application categories.

Second, Waze exhibits the strongest classification performance, with the highest concentration of correctly classified flows along the diagonal. This observation agrees with the classification report, where Waze achieved the highest F1-score (0.9355), suggesting that its traffic possesses highly distinctive characteristics.

Third, although the diagonal dominates the matrix, several off-diagonal entries are visible. These represent misclassified flows and occur primarily among applications with similar traffic behavior, such as social networking, messaging and general internet applications. Such overlap is expected because many mobile applications generate comparable packet sizes, flow durations and inter-arrival time distributions.

From a QoS perspective, the confusion matrix indicates that applications with distinctive traffic characteristics can be reliably identified, enabling appropriate QoS policy assignment. Conversely, applications exhibiting overlapping traffic patterns may occasionally receive an incorrect QoS classification. Nevertheless, the relatively limited number of off-diagonal errors demonstrates that the proposed framework maintains good generalization performance when evaluated on previously unseen network traffic.

Overall, the confusion matrix confirms that the proposed Random Forest classifier provides a reliable basis for application-aware QoS policy mapping while avoiding the optimistic bias associated with evaluating models on oversampled test data.

- 4.4. Feature Importance Analysis

The Random Forest model identified the most discriminative features for traffic classification. Feature importance analysis reveals which flow-level statistical characteristics are most valuable for distinguishing between different application types. Besides improving classification performance, understanding these features provides valuable insight into application behavior and supports network monitoring and traffic management [9]. After training the model, the Random Forest classifier identified the following 10 important features contributing to classification accuracy [24].

To understand why these features matter for QoS traffic classification, Table 7 summarizes the top 10 features along with their traffic behavior implications and corresponding QoS relevance. The top-ranked features indicate that packet length and inter-arrival time (IAT) statistics play the most dominant role in differentiating application behaviors. This aligns with findings from Aceto et al. [16], which emphasize that packet size variability and timing patterns are key discriminants between real-time and background mobile applications.

The results indicate that packet length statistics and inter-arrival time (IAT) measurements dominate the most informative features, demonstrating that packet size characteristics and temporal communication patterns are the strongest discriminators between mobile application types. These findings are consistent with Aceto et al. [16], who reported that packet size distributions and timing behavior remain among the most discriminative flow-level characteristics for encrypted mobile traffic classification.

Packet length features capture differences in the amount of information exchanged during communication sessions, whereas inter-arrival time features characterize how frequently packets are transmitted. Streaming applications such as YouTube and Spotify typically generate larger and more regular packets to sustain continuous media delivery. Conversely, interactive applications such as Viber, Messenger, Waze, and Slither exhibit shorter packet intervals due to frequent bidirectional communication, while productivity and background synchronization applications generally display longer inter-arrival times and less stringent timing requirements.

These observations also provide a practical justification for the rule-based QoS policy mapping proposed in this study. Rather than assigning QoS policies arbitrarily, the mappings were informed by traffic behavior revealed through feature importance analysis together with established QoS service-class recommendations described in RFC 4594. Applications exhibiting frequent packet transmission and stringent timing requirements were assigned higher-priority, lower-latency QoS policies, whereas applications characterized by larger but less time-sensitive data transfers were assigned medium- or lower-priority policies with greater tolerance for delay and jitter.

Although this study evaluates representative mobile application traffic rather than operational 5G traffic, the results demonstrate that flow-level statistical features can accurately distinguish application behavior and provide the information required for application-aware QoS decision-making. Consequently, the proposed framework illustrates how machine learning-based traffic classification can serve as a foundation for future QoS management mechanisms in next-generation mobile networks, where classified application traffic may subsequently be mapped to standardized QoS profiles such as the 5G QoS Identifier (5QI).

The Random Forest model’s feature importance thus serves not only for accurate classification but also for QoS policy enforcement. This bridges the gap between traffic analysis and network resource optimization.

Table 7. Feature importance.

| Feature Name | Traffic Behavior Interpretation | QoS implication |
| --- | --- | --- |
| packet_length_ upstream_flow _max | Maximum packet size in uplink traffic (e.g., video calls or uploads). | Large uplink packets require low-latency and high-throughput uplink allocation. |
| packet_length_ upstream_flow _var | Variance of uplink packet sizes; emphasizes extreme differences. | Identifies apps switching between control and payload packets. |
| packet_length_ upstream_flow _std | Variation in uplink packet sizes; distinguishes browsing vs. streaming. | High variability signals adaptive bitrate streaming; needs burst capacity. |
| packet_length_ upstream_flow _90_percentile | Typical large packet size in uplink. | Indicates consistent media chunking; requires smooth uplink QoS. |
| packet_length_ downstream_fl ow_max | Largest packet size in downlink. | Streaming apps (YouTube); high-downlink throughput demand. |
| packet_length_ biflow_max | Maximum packet size across both directions. | Captures overall session intensity (video vs. browsing). |
| iat_upstream_flow_max | Longest time gap between uplink packets. | Low gaps = real-time apps (e.g., calls), high gaps = background sync apps. |
| iat_biflow_max | Longest time gap across both directions. | Distinguishes continuous vs. idle traffic; adaptive scheduling needed. |
| packet_length_ upstream_flow _mean | Average downlink packet size. | Distinguishes text/chat apps from video or file transfer. |
| iat_downstrea m_flow_max | Longest delay between downlink packets. | Identifies idle/background traffic vs. continuous streaming. |

- 4.5. Model Performance Under Different Configurations

The model was trained and evaluated using flow-level statistical features extracted from the MIRAGE-2019 dataset. The evaluation compared five different pre-processing configurations to assess the impact of oversampling and dimensionality reduction techniques on the Random Forest classifier’s performance. Each experiment used an 80:20 train–test split with stratification to preserve class balance.

Five scenarios were considered:

- Baseline (no oversampling, no dimensionality reduction);

- SMOTE (Synthetic Minority Oversampling Technique);

- SMOTE and LDA (combining oversampling with dimensionality reduction);

- RandomOverSampler (simple oversampling by duplication);

- RandomOverSampler and LDA.

The key metrics recorded were Accuracy, Macro F1-Score and Weighted F1-Score, alongside the total running time. These metrics provide a balanced evaluation of overall correctness, per-class fairness, and computational cost.

The results are summarized in Table 8.

Table 8. Performance comparison.

| Configuration | Accuracy | Macro F1-Score | Weighted F1-Score | Training Time | Remarks |
| --- | --- | --- | --- | --- | --- |
| No Oversampling, No LDA | 0.7166 | 0.6683 | 0.72 | ~59s | Baseline model; suffers from class imbalance. |
| SMOTE | 0.7175 | 0.6686 | 0.7225 | ~3 min and 13s | Significant Accuracy gains but high computation time due to synthetic data generation. |
| SMOTE and LDA | 0.6174 | 0.5698 | 0.6233 | ~1 min and 47s | LDA reduced features but slightly lowered Accuracy, and improved stability. |
| RandomOverSampler | 0. 7183 | 0. 6701 | 0. 7227 | ~2 min and 21s | Achieved best Accuracy and balance between Recall and Precision. |
| RandomOverSampler and LDA | 0.6205 | 0.5722 | 0.6227 | ~1 min and 38s | Drop in Aaccuracy, but best trade-off between Accuracy and computational cost. |

The results demonstrate that oversampling substantially improves the model’s ability to classify minority classes in the imbalanced MIRAGE dataset. Overall, oversampling produced only a modest improvement in classification performance, while dimensionality reduction using Linear Discriminant Analysis (LDA) consistently reduced computational cost at the expense of predictive accuracy.

            - No oversampling, no dimension reduction: Without oversampling or dimensionality reduction, the Random Forest classifier achieved an Accuracy of 71.66%, a Macro F1-score of 0.6683, and a Weighted F1-score of 0.72. This configuration served as the baseline model and demonstrated the effect of class imbalance, where majority application classes exerted greater influence during model training, resulting in lower performance for minority classes.

- SMOTE: Applying the Synthetic Minority Oversampling Technique (SMOTE) produced a slight improvement in performance, increasing the Accuracy to 71.75%, the Macro F1-score to 0.6686, and the Weighted F1-score to 0.7225. Although SMOTE helped balance the class distribution by generating synthetic minority samples, the improvement over the baseline was marginal while substantially increasing the training time to approximately 3 min and 13s. The additional computational overhead results from the generation of synthetic observations and the larger training dataset.

- SMOTE and LDA: Combining SMOTE with LDA reduced the feature dimensionality before classification, decreasing the training time to approximately 1 min and 7s. However, classification performance declined considerably, achieving an Accuracy of 61.74%, a Macro F1-score of 0.5698, and a Weighted F1-score of 0.6233. Although dimensionality reduction reduced computational cost, the transformation compressed discriminative information required distinguishing between similar application traffic flows, resulting in reduced predictive performance.

- RandomOverSampler Performance: RandomOverSampler achieved the highest overall performance among all evaluated configurations, obtaining an Accuracy of 71.83%, a Macro F1-score of 0.6701, and a Weighted F1-score of 0.7227. Unlike SMOTE, RandomOverSampler balances the dataset by duplicating existing minority-class samples rather than generating synthetic examples, thereby preserving the original feature distribution. This approach produced the most balanced Precision and Recall across application classes while requiring approximately 2 min and 21s for training.

- RandomOverSampler and LDA: Applying LDA after RandomOverSampler reduced the training time further to approximately 1 min and 38s. However, classification Accuracy decreased to 62.05%, with a Macro F1-score of 0.5722 and a Weighted F1-score of 0.6227. Although this configuration offered the lowest computational cost among the oversampling approaches, the reduction in predictive performance indicates that LDA removed information useful for distinguishing between application classes.

Overall, the experimental comparison indicates that oversampling alone provides only modest improvements in classification performance for the MIRAGE-2019 dataset under the corrected experimental workflow. Among the evaluated techniques, RandomOverSampler without dimensionality reduction achieved the best balance between classification accuracy and class-wise performance, whereas LDA consistently reduced training time but introduced a noticeable reduction in predictive accuracy. Consequently, RandomOverSampler was selected as the preferred pre-processing technique for the proposed framework because it achieved the highest overall classification performance while preserving the original feature representation.

4.6. Cross-Validation

Cross-validation is a model evaluation technique in which the dataset is partitioned into k subsets (folds). The model is trained on k − 1 folds and evaluated on the remaining fold, with the process repeated until every fold has been used for testing. The final performance is obtained by averaging the results across all folds, providing a more reliable estimate of model generalization than a single train/test split.

During the experimentation phase, attempts to perform cross-validation with SMOTE and Random Forest led to memory allocation failures. The MIRAGE-2019 dataset contains more than 120,000 bidirectional network flows and over 100 statistical features. Applying SMOTE within each fold substantially increased the size of the training data, while Random Forest required the construction of hundreds of decision trees for every fold. For example, if you use 5-fold cross-validation, the training process repeats five times on large, oversampled data. Random Forest itself is memory intensive because it builds hundreds of trees, each storing feature splits, thresholds, and intermediate results. The repeated resampling and model training exceeded the available system memory, preventing successful completion of cross-validation.

To avoid data leakage, the final experimental workflow first partitioned the original dataset into training and testing subsets using a stratified train/test split. Feature standardization was fitted only on the training data and then applied to the testing data. RandomOverSampler or SMOTE was subsequently applied only to the training set, while the testing set remained completely untouched. This ensured that no duplicated or synthetic samples were present in the testing data and prevented information leakage between the training and evaluation stages.

Although a stratified hold-out validation strategy provides an unbiased estimate of model performance when implemented correctly, it is generally less robust than repeated stratified cross-validation because the evaluation depends on a single data partition. Consequently, the reported classification Accuracy and F1-score should be interpreted within the context of this validation strategy.

Together, these three factors led to memory exhaustion, especially when operating on large flow-level datasets like MIRAGE-2019 which has +120 000 flows and +100 features.

Future work will address this challenge by exploring:

Mini-batch or incremental training;

Memory-efficient data generators;

GPU-accelerated learning environments;

Dimensionality reduction prior to resampling.

4.7. Comparative Evaluation with Related Studies

To conceptualize the performance of the system, results were compared with recent related works that implemented supervised machine learning for network traffic classification and QoS optimization. The comparative evaluations are summarized in Table 9.

Table 9. Comparative evaluations with two other studies.

| Study | Algorithm | Dataset | Accuracy (%) | Remarks |
| --- | --- | --- | --- | --- |
| Mohammed & Ghani (2025) [5] | RF + SMOTE | 5G Dataset | 90.1 | Traffic classification for QoS optimization |
| Ganesan et al. (2021) [7] | RF | IoT Dataset | 87.0 | SDN-enabled traffic classification |
| This study (2025) | RF + ROS | MIRAGE2019 | 71.8 | Traffic classification plus QoS Policy Mapping |

The studies summarized in this table use different datasets, traffic environments, feature sets and evaluation protocols. Therefore, the reported performance values should be interpreted as contextual comparisons rather than direct one-to-one benchmarks.

Mohammed and Ghani [5] developed a Random Forest classifier using a labeled 5G traffic dataset to improve QoS optimization. Their system achieved an Accuracy of 90.1% with the integration of SMOTE improving minority class representation. In contrast, the present study uses the publicly available MIRAGE-2019 mobile traffic dataset together with RandomOverSampler to address class imbalance and achieved an overall Accuracy of 71.8%. Although the classification accuracy is lower, the objective of this study differs from that of Mohammed and Ghani [5]. Rather than focusing solely on classification performance, this work integrates a rule-based QoS policy mapping component that translates the predicted application classes into network management recommendations, including application priority, bandwidth allocation, latency sensitivity and jitter tolerance. This demonstrates how machine learning classification can support QoS-aware decision-making in future mobile network environments.

Ganesan et al. [7] applied various supervised ML models, including Random Forest and SVM, within an SDN-enabled FiWi-IoT environment. Their Random Forest achieved approximately 87% Accuracy with moderate training cost. Their work primarily focused on evaluating the suitability of machine learning algorithms for software-defined network management. By comparison, the proposed framework extends beyond traffic classification by incorporating application-aware QoS policy mapping that associates each classified application with representative QoS requirements. This provides a practical mechanism for illustrating how classification outputs can support differentiated network resource allocation.

Overall, the comparative evaluation demonstrates that Random Forest remains a robust and competitive algorithm for encrypted network traffic classification. While the proposed framework achieves a lower classification accuracy than some existing studies, it introduces an additional application-aware QoS policy mapping stage that bridges the gap between traffic classification and practical QoS management. Furthermore, the use of a publicly available mobile application dataset contributes to the reproducibility of the proposed framework and provides a foundation for future evaluation using native 5G traffic datasets.

Recent ensemble methods such as XGBoost and LightGBM have demonstrated competitive performance in network traffic classification. However, this study selected Random Forest because it is one of the most widely adopted benchmark algorithms in the literature, provides strong interpretability through feature importance analysis, requires relatively limited parameter tuning, and enables direct comparison with previous QoS-oriented studies. Evaluating gradient boosting methods remains an important direction for future work.

4.8. QOS Policy Analysis

Figure 3 depicts the distribution of network traffic according to the assigned QoS priority levels. The figure illustrates the proportion of application flows classified as High, Medium, and Low Priority based on the proposed rule-based QoS policy mapping. This distribution provides insight into the resource requirements of different application categories and supports the design of QoS-aware traffic management policies.

Figure 3. Traffic distribution by QOS priority.

The Medium-Priority category represents the largest proportion of the dataset, accounting for 48.1% (58,650 flows). These applications generally require balanced network performance with moderate bandwidth and latency requirements. Examples include social networking, e-commerce, media streaming, and general web-based applications. Since this category constitutes nearly half of all traffic, efficient resource allocation for Medium-Priority services is essential to maintain overall network performance while ensuring fair utilization of available bandwidth.

Low-Priority traffic accounts for 28.2% (34,385 flows) of the total network flows. These applications are generally delay tolerant and include background synchronization, cloud storage, productivity applications, and other services that can tolerate higher latency and jitter without significantly affecting the user experience. Such traffic can therefore be scheduled with lower precedence during periods of network congestion.

High Priority traffic represents 23.7% (28,920 flows) of the dataset and corresponds to latency-sensitive and interactive applications such as navigation, voice communication, and online gaming. These applications require rapid packet delivery, minimal delay variation (jitter), and reliable connectivity to maintain acceptable Quality of Service. Consequently, they should receive preferential treatment during network scheduling and resource allocation.

Overall, the observed priority distribution reflects a realistic mobile application environment in which most traffic consists of services requiring moderate QoS guarantees, while a smaller proportion comprises highly delay-sensitive applications that demand preferential network treatment.

Figure 4 presents the distribution of network traffic flows according to their latency sensitivity levels, which directly influence Quality of Service (QoS) requirements in future 5G networks. The figure illustrates the proportion of application traffic requiring different levels of delay tolerance, providing insight into how network resources could be prioritized to support application-aware QoS policies.

Figure 4. Traffic distribution by latency sensitivity.

Medium latency sensitivity constitutes the largest proportion of the traffic, accounting for 53.5% (65,265 flows). This category represents applications that require stable network performance but can tolerate moderate transmission delays, such as social media, location-based services, and multimedia streaming. Although these applications do not demand ultra-low latency, they benefit from consistent throughput and reliable connectivity to maintain an acceptable quality of experience.

Low latency sensitivity accounts for 28.2% (34,385 flows). These flows typically correspond to delay-tolerant services such as cloud storage, file synchronization, downloads, productivity applications, and background data transfers. Since these applications are less affected by transmission delays, they can be assigned lower scheduling priority without significantly affecting user experience, allowing network resources to be allocated to more latency-sensitive services when congestion occurs.

High latency sensitivity represents 15.7% (19,116 flows) of the total traffic. This category includes interactive applications such as voice over IP (VoIP), messaging, and navigation services where communication delays have a direct impact on responsiveness and user experience. These applications require preferential scheduling and lower end-to-end delays to ensure consistent real-time performance.

Very-high latency sensitivity constitutes the smallest proportion of the dataset, accounting for only 2.6% (3189 flows). Although this represents a relatively small percentage of the overall traffic, these applications have the strictest Quality of Service requirements. They include highly interactive applications such as online gaming (e.g., Slither.io), where minimal latency and jitter are essential for maintaining responsiveness. Consequently, these flows should receive the highest priority during QoS resource allocation despite their comparatively low traffic volume.

Overall, the distribution indicates that while 81.7% of application traffic (medium and low sensitivity) can tolerate some degree of transmission delay, approximately 18.3% of flows require high or very high responsiveness. This demonstrates the importance of accurately classifying application traffic before QoS policies are applied, enabling network operators to prioritize delay-sensitive applications while efficiently allocating network resources to delay-tolerant traffic.

The distribution of application traffic according to bandwidth requirements is presented in Figure 5. The figure illustrates the proportion of classified application flows that require high, medium, low, or variable bandwidth to support their expected Quality of Service (QoS).

Figure 5. Traffic distribution by bandwidth requirements.

Medium bandwidth traffic represents the largest proportion of the dataset, accounting for 54.2% (66,065 flows). This category includes applications such as social media platforms, navigation services, music streaming, and several general-purpose mobile applications that require consistent throughput to maintain satisfactory user experience without demanding extremely high transmission rates. The dominance of this category reflects the prevalence of everyday mobile applications that require moderate but sustained network resources.

Low bandwidth traffic constitutes 36.2% (44,192 flows). Applications within this category include messaging services, weather applications, productivity tools, and other lightweight applications that exchange relatively small amounts of data. These applications typically operate effectively under constrained network conditions and therefore impose comparatively low bandwidth demands on the network.

High bandwidth traffic accounts for 5.3% (6493 flows). This category consists primarily of bandwidth-intensive applications such as video streaming and certain interactive multimedia services that require higher throughput to maintain acceptable performance and user experience. Although these applications represent a relatively small proportion of the overall traffic, they consume a significant share of available network resources and therefore require appropriate QoS mechanisms to ensure efficient bandwidth allocation.

Variable bandwidth traffic represents 4.3% (5205 flows). This category includes applications such as cloud storage and file synchronization services whose bandwidth requirements fluctuate according to user activity, background synchronization processes, and file transfer operations. Consequently, these applications exhibit dynamic network resource demands rather than maintaining a fixed bandwidth profile.

Overall, the bandwidth distribution demonstrates that most applications’ traffic falls within the medium and low bandwidth categories, accounting for approximately 90.4% of all classified flows. These findings support the proposed QoS policy mapping by illustrating that different application classes exhibit distinct bandwidth requirements, enabling differentiated resource allocation based on application characteristics. The bandwidth distribution, together with the priority and latency analyses, provides a comprehensive representation of the network demand profile and demonstrates how the proposed classification framework can support QoS-aware traffic management in future mobile network environments.

QOS Policy Recommendations

The primary contribution of this study is the accurate classification of application-level network traffic using a Random Forest classifier. To demonstrate the practical applicability of the classification results, a rule-based QoS policy mapping framework was developed to translate the predicted application classes into representative Quality of Service (QoS) recommendations. Rather than learning QoS policies directly from the dataset, the framework assigns QoS attributes based on the functional communication characteristics of each application.

The QoS mappings were informed by the service-class recommendations described in RFC 4594, Configuration Guidelines for DiffServ Service Classes [25]. RFC 4594 classifies network applications according to their latency, jitter, packet loss, and bandwidth requirements to support differentiated network services. Although RFC 4594 was developed for differentiated service (DiffServ) networks rather than 5G networks, the underlying QoS principles remain applicable to application-aware traffic management and continue to influence modern QoS architectures. Consequently, RFC 4594 provides a suitable standards-based foundation for demonstrating how application-aware traffic classification can be translated into QoS recommendations within a 5G-oriented framework.

Applications were first grouped according to their primary communication characteristics, including telephony, multimedia streaming, real-time interactive services, transactional web applications, cloud data transfer, and standard best-effort traffic. Each application category was then associated with the closest DiffServ service class defined in RFC 4594. Finally, the selected service class was used to assign four QoS attributes within the proposed framework, namely priority level, bandwidth allocation, latency sensitivity, and jitter tolerance.

It should be noted that these mappings are intended as a standards-informed decision-support mechanism rather than operator-specific QoS configurations (see Table 10). Modern 5G networks implement differentiated QoS through standardized 5G QoS Identifiers (5QIs) defined by the 3GPP architecture. In this study, RFC 4594 provides the rationale for application-level QoS characteristics, while the proposed framework demonstrates how machine learning-based traffic classification can provide the application awareness required for future integration with 5G QoS mechanisms.

Table 10. QoS mappings with service class.

| Application Category | RFC 4594 Service Class | Example Applications | Assigned QoS Characteristics |
| --- | --- | --- | --- |
| Telephony | Telephony | Viber, Messenger | High priority, low jitter, high latency sensitivity |
| Interactive Real-Time | Interactive Real-Time | Waze, Slither.io | High priority, medium bandwidth, very high latency sensitivity |
| Multimedia Streaming | Multimedia Streaming | YouTube, Spotify | Medium priority, high/medium bandwidth, moderate latency sensitivity |
| Transactional Data | Transactional Data | Wish,Groupon, Subito | Medium priority, medium bandwidth, moderate latency sensitivity |
| Standard Best-Effort | Standard | Facebook,Twitter, Pinterest,Duolingo, AccuWeather, TripAdvisor, FourSquare, Iliga | Medium or low priority, delay tolerant |
| Bulk Data Transfer | Bulk Data | Dropbox | Low priority, variable bandwidth, high jitter tolerance |
| Productivity | Standard | Trello | Low priority, low bandwidth, delay tolerant |

The proposed framework extends beyond application-level traffic classification by incorporating a rule-based QoS policy mapping module that generates application-aware QoS recommendations for each predicted traffic class. Rather than assigning arbitrary QoS policies, the framework derives these recommendations from the communication characteristics of each application using the DiffServ service class guidelines presented in RFC 4594.

Following classification by the Random Forest model, each predicted application is first associated with an appropriate application category, such as Telephony, Multimedia Streaming, Interactive Real-Time, Transactional Data, Bulk Data Transfer, or Standard Best-Effort traffic. Each category is then mapped to the closest RFC 4594 service class, from which the corresponding QoS attributes are assigned, including priority level, bandwidth allocation, latency sensitivity, and jitter tolerance. This standards-informed mapping provides a structured and reproducible method for translating machine learning predictions into practical QoS recommendations.

The resulting QoS recommendations demonstrate the practical applicability of the proposed traffic classification framework beyond merely identifying application types. By transforming predicted application labels into standardized QoS policies, the framework provides a decision-support layer that could be integrated into software-defined networking (SDN) or future 5G network management systems to support application-aware resource allocation, traffic prioritization, and differentiated service provisioning.

Although RFC 4594 provides the basis for defining application service classes, modern 5G networks implement differentiated Quality of Service through standardized 5G QoS Identifiers (5QIs) specified by the 3GPP architecture. Accordingly, the proposed framework should not be interpreted as a direct implementation of 5G QoS scheduling. Instead, it demonstrates how machine learning-based traffic classification can provide the application awareness required for future integration with standardized 5G QoS mechanisms.

5. Practical Implications

The findings of this study extend beyond classification accuracy by demonstrating how machine learning-based traffic classification can support application-aware Quality of Service (QoS) management. Although the proposed framework was evaluated using a representative mobile traffic dataset rather than operational 5G traffic, it illustrates how application classification can provide the information required for intelligent QoS decision-making in future 5G and beyond-5G networks.

5.1. Network Management Benefits

The proposed framework demonstrates several practical benefits for application-aware network management. By automatically classifying encrypted application traffic and mapping the predicted application class to predefined QoS policies, the framework reduces the manual effort required to configure application-specific network rules. The QoS mappings are rule-based and were developed according to the typical communication characteristics of the applications, informed by established QoS service-class guidelines such as RFC 4594.

Because it relies only on flow-level statistical features, it preserves user privacy and remains compliant with regulations such as the GDPR (General Data Protection Regulation) [26], enabling accurate traffic recognition even when payloads are encrypted. This makes the approach compatible with modern encrypted communication protocols where Deep Packet Inspection is no longer practical.

The Random Forest classifier also offers computational efficiency and scalability through parallel tree construction, making it suitable for analyzing large numbers of network flows. Accurate application identification enables network operators to associate applications with appropriate QoS requirements, including latency sensitivity, bandwidth demand, and jitter tolerance. Such information could support future QoS-aware resource allocation mechanisms within software-defined networking (SDN), Network Function Virtualization (NFV), and next-generation mobile network architectures.

It should be noted, however, that the proposed framework demonstrates the feasibility of application-aware QoS mapping rather than implementing QoS enforcement within a live 5G network.

5.2. Deployment Considerations

Deploying such a system requires careful consideration of several operational factors. Model inference and feature extraction introduce additional CPU and memory overhead, meaning production environments may need GPU acceleration or offloading to edge nodes to maintain performance at scale. Because application traffic patterns evolve as software updates, protocols change, and user behavior shifts, the model must be periodically retrained with fresh data to preserve accuracy over time. Effective deployment also depends on dynamic policy management: The current QoS policy mapping is implemented using predefined rule-based mappings derived from application characteristics. In operational environments, these mappings would ideally integrate with existing network management frameworks, such as SDN controllers, NFV orchestrators, or standardized 3GPP QoS mechanisms (e.g., 5QI), allowing QoS policies to be dynamically assigned based on current network conditions, operator policies, and service-level agreements.

Accordingly, the framework should be viewed as a proof of concept demonstrating how machine learning classification outputs can support automated QoS decision-making rather than a complete deployment-ready network management solution.

5.3. Limitations and Challenges

The study faces several dataset-related limitations, primarily because MIRAGE-2019 captures traffic patterns from 2017 to 2019, before widespread 5G deployment. Consequently, the dataset does not capture important characteristics of modern 5G environments, including network slicing, ultra-reliable low-latency communications (URLLC), massive machine-type communications (mMTC), or traffic generated using more recent protocols such as TLS 1.3 and QUIC. Therefore, this work should be interpreted as a proxy study using representative labeled mobile application traffic rather than an evaluation performed on operational 5G network traffic. Most existing work relies on simulations or costly proprietary data, so MIRAGE-2019 was used as the closest open alternative. Its traffic was collected in a single academic environment in Italy, meaning the flows may not generalize to diverse networks, devices, or geographical regions. The study also uses only 20 of the 40+ available applications, simplifying training but limiting coverage of niche, evolving, or newly emerging apps.

From a methodological perspective, the QoS policy mapping is rule-based rather than learned from network observations or validated using operational QoS measurements. The mappings demonstrate how application classification can be translated into QoS decisions but should not be interpreted as universal QoS policies applicable to all networks.

While the overall classifier achieved satisfactory performance, several applications including Groupon, Trello, TripAdvisor and Viber produced comparatively low F1-scores. Because QoS policies are assigned directly from the predicted application class, misclassification of latency-sensitive applications may lead to inappropriate resource allocation. For example, incorrectly classifying Viber traffic may result in a lower-priority QoS policy being assigned, potentially increasing latency or jitter for delay-sensitive voice communication. Consequently, the proposed QoS mapping should be viewed as a proof of concept demonstrating the integration of machine learning classification with application-aware policy selection rather than a production-ready autonomous QoS controller. Future work should improve minority-class classification before deployment in operational networks.

Finally, although the Random Forest classifier achieved strong classification performance, the proposed framework operates offline using previously captured traffic traces. Additional work is required to evaluate inference latency, computational efficiency, and integration with real-time network management platforms before deployment in operational 5G environments.

6. Conclusion and Future Work

This study presented a supervised machine learning framework for application-level network traffic classification using statistical flow-level features extracted from the MIRAGE-2019 dataset. The classified applications were subsequently mapped to predefined QoS policies to demonstrate how machine learning can support application-aware QoS decision-making in future mobile networks.

Key contributions include:

A supervised machine learning framework for application-level traffic classification using Random Forest;

A rule-based QoS policy mapping framework, informed by established QoS service-class guidelines, demonstrating how classification outputs can support application-aware QoS management;

A privacy-preserving approach which demonstrated the ability to classify encrypted traffic using only flow-level features, eliminating the need for payload inspection while preserving user confidentiality;

A comprehensive pre-processing pipeline incorporating feature scaling, oversampling, and optional dimensionality reduction;

A reproducible implementation that integrates data aggregation, pre-processing, model training, evaluation, visualization, and QoS policy assignment.

Compared with prior studies- such as Ganesan et al. [7], who reported approximately 87% Accuracy using Random Forest in the SDN-based environment, and Mohammed & Ghani [5], who achieved 90% using Random Forest- the proposed approach achieves comparable classification performance while demonstrating the practical integration of machine learning classification with QoS policy mapping. However, unlike operational QoS management systems, the proposed framework serves as a proof of concept illustrating how classified application traffic may support future QoS-aware resource allocation.

A limitation of this study is that model evaluation was based on a single stratified 80:20 train–test split rather than repeated cross-validation. Although this approach prevented data leakage and preserved class distributions, the reported performance estimates may vary across different data partitions. Attempts to combine repeated cross-validation with oversampling and Random Forest exceeded the available memory resources because of the large number of flow records and high-dimensional feature space. Future work will investigate more memory-efficient validation strategies such as pipeline-based resampling, stratified k-fold validation without oversampling, or evaluation on reduced representative subsets.

Because the evaluation was conducted using the MIRAGE-2019 dataset, which pre-dates commercial 5G deployment, the findings should be interpreted as demonstrating the feasibility of the proposed classification framework on representative mobile application traffic rather than validating performance within live 5G networks.

6.1. Future Work

Future work focuses on improving both the technical capability and practical deployment of the system. Technically, the model should be extended to support real-time traffic classification and evaluated using more advanced machine learning methods such as Gradient Boosting, XGBoost, CNNs, and LSTMs to improve generalization. System enhancements include introducing adaptive QoS control using reinforcement learning which should allow QoS policies to self-adjust based on feedback, incorporating richer QoS metrics like jitter variance, throughput and packet loss, enabling context-aware decision-making based on live network conditions, and supporting automatic model updates through online learning. Evaluation should be expanded by testing the model on multiple datasets, analyzing long-term model drift which includes tracking how the accuracy and performance changes over time as applications and protocols evolve, identifying optimal retraining intervals. Future evaluation should incorporate memory-efficient stratified K-fold cross-validation with oversampling performed within each training fold to provide more robust estimates of model generalization while avoiding data leakage. Evaluation will include assessing user quality of experience in real deployments and contributing to modern 5G-labeled datasets. Finally, standardization and industry alignment are key: integrating with 3GPP QoS standards, releasing the toolkit as open source, and collaborating with mobile operators to support deployment in real 5G network environments.

6.2. Acknowledgement

The authors would like to express sincere gratitude to the University of KwaZulu-Natal for financial support that has made this work possible. We also thank the department of computer science for availing resources that contributed to the development, refinement, and completion of this work.

6.3. Data and Code Availability

The implementation developed for this study is publicly available on GitHub (Release v1.0.0) GitHub Repository: https://github.com/tomRiddle-the1st/network_traffic_classification The repository contains the complete source code, pre-processing scripts, model training pipeline, QoS policy mapping implementation, evaluation scripts, visualization code, and README documentation, required to reproduce the experiments.

Due to GitHub file size limitations, large project files (including the processed datasets, trained models, feature scaler, optional LDA transformer, generated charts, and supplementary project files) are hosted separately on Google Drive. Google Drive (Dataset and Supplementary Materials): https://drive.google.com/drive/folders/1mQd6-YC9lhvdlCPILjPQdQCWuWPrW8az?usp=sharing (08 July 2026).

The Google Drive folder contains all supplementary project resources required for reproducibility. The repository README and user manual describe the project structure and explain the contents of each directory.

The experiments in this study are based on the publicly available MIRAGE-2019 mobile traffic dataset. Instructions for obtaining the original dataset are provided in the repository README together with the appropriate citation to the original dataset authors.

Supplementary Materials: The following supporting information can be downloaded at https://www.mdpi.com/article/doi/s1.

Author Contributions: Conceptualization, M.A.I. and O.P.K.; methodology, M.A.I. and O.P.K.; software, M.A.I.; validation, M.A.I. and O.P.K.; formal analysis, M.A.I. and O.P.K.; investigation, M.A.I.; resources, M.A.I. and O.P.K.; data curation, M.A.I.; writing- original draft preparation, M.A.I.; writing-review and editing, M.A.I. and O.P.K.; visualization, M.A.I.; supervision, O.P.K.; project administration, O.P.K.; funding acquisition, O. P. K. All authors have read and agreed to the published version of the manuscript.

Funding: This research received no external funding.

Data Availability Statement: The original contributions presented in this study are included in the article. Further inquiries can be directed to the corresponding authors.

Conflicts of Interest: The authors declare no conflict of interest.

Abbreviations

The following abbreviations are used in this manuscript:

4G		Fourth Generation Network

5G		Fifth Generation Network

QoS		Quality of Service

eMBB		enhanced Mobile Broadband

URLLC	Ultra-Reliable Low-Latency Communications

mMTC	massive Machine-Type Communications

ISP		Internet Service Provider

DPI		Deep Packet Inspection

References

Azab, A.; Khasawneh, M.; Alrabaee, S.; Choo, K.-K.R.; Sarsour, M. Network traffic classification: Techniques, datasets, and challenges. Digit. Commun. Netw. 2024, 10, 676–692. https://doi.org/10.1016/j.dcan.2022.09.009.

Maranon, T; Armando, J. Traffic Classification of 5G Packet Traces. Chalmers University of Technology. 2023. Master's Thesis. Computer systems and networks (MPCSN), MSc. Available online: https://hdl.handle.net/20.500.12380/307492. (accessed on 24 April 2025)

Canever, H.; Wang, X. Network Traffic Classification Using Unsupervised Learning: A Comparative Analysis of Clustering Algorithms. 2023. Available online: https://hal.science/hal-04149117/ (accessed on 23 April 2025).

Mahmood, M.; Khalil, S.A.; Shah, S.J.; Ahmad, M. Network Traffic Classification Techniques and Comparative Evaluation of Machine Learning Models. Int. J. Innov. Sci. Res. Technol. 2023, 8, 1135–1139.

Mohammed, A.Q.; Ghani, R.F. Network traffic classification to improve quality of service (QoS). In AIP Conference Proceedings; AIP Publishing: Baghdad, Iraq, 2025; p. 020007. https://doi.org/10.1063/5.0264880.

Erman, J.; Arlitt, M.; Mahanti, A. Traffic classification using clustering algorithms. In Proceedings of the 2006 SIGCOMM Workshop on Mining Network Data; ACM: Pisa, Italy, 2006; pp. 281–286. https://doi.org/10.1145/1162678.1162679.

Ganesan, E.; Hwang, I.-S.; Liem, A.T.; Ab-Rahman, M.S. SDN-Enabled FiWi-IoT Smart Environment Network Traffic Classification Using Supervised ML Models. Photonics 2021, 8, 201. https://doi.org/10.3390/photonics8060201.

Karim, S.; He, H.; Laghari, A.A.; Madiha, H. Quality of Service (QoS): Measurements of Video Streaming. arXiv 2019, arXiv:2008.12017. https://doi.org/10.5281/zenodo.3987056.

Nguyen, T.T.T.; Armitage, G. A survey of techniques for internet traffic classification using machine learning. IEEE Commun. Surv. Tutor. 2008, 10, 56–76. https://doi.org/10.1109/SURV.2008.080406.

Shafiq, M.; Yu, X.; Laghari, A.A.; Yao, L.; Karn, N.K.; Abdessamia, F. Network Traffic Classification techniques and comparative analysis using Machine Learning algorithms. In 2016 2nd IEEE International Conference on Computer and Communications (ICCC); IEEE: Piscataway, NJ, USA, 2016; pp. 2451–2455. https://doi.org/10.1109/CompComm.2016.7925139.

Moore, A.W.; Papagiannaki, K. Toward the Accurate Identification of Network Applications. In International Workshop on Passive and Active Network Measurement; Springer: Berlin/Heidelberg, Germany, 2005. Available online: https://www.researchgate.net/publication/220850310_Toward_the_Accurate_Identification_of_Network_Applications (accessed on 12 July 2025).

Madhukar, A.; Williamson, C. A Longitudinal Study of P2P Traffic Classification. In 14th IEEE International Symposium on Modeling, Analysis, and Simulation, Monterey, CA, USA; IEEE: Piscataway, NJ, USA, 2006. https://doi.org/10.1109/mascots.2006.6.

Liao, M.-Y.; Luo, M.-Y.; Yang, C.-S.; Chen, C.-H.; Wu, P.-C.; Chen, Y.-W. Design and evaluation of deep packet inspection system: A case study. IET Netw. 2012, 1, 2–9. https://doi.org/10.1049/iet-net.2011.0048.

Labayen, V.; Magaña, E.; Morató, D.; Izal, M. Online classification of user activities using machine learning on network traffic. Comput. Netw. 2020, 181, 107557. https://doi.org/10.1016/j.comnet.2020.107557.

Erman, J.; Mahanti, A.; Arlitt, M.; Cohen, I.; Williamson, C. Semi-supervised network traffic classification. In Proceedings of the 2007 ACM SIGMETRICS International Conference on Measurement and Modeling of Computer Systems; ACM: San Diego, CA, USA, 2007; pp. 369–370. https://doi.org/10.1145/1254882.1254934.

Aceto, G.; Ciuonzo, D.; Montieri, A.; Persico, V.; Pescape, A. MIRAGE: Mobile-app Traffic Capture and Groundtruth Creation. In 2019 4th International Conference on Computing, Communications and Security (ICCCS), Rome, Italy; IEEE: Piscataway, NJ, USA, 2019; pp. 1–8. https://doi.org/10.1109/CCCS.2019.8888137.

Choi, Y.-H.; Kim, D.; Ko, M.; Cheon, K.-Y.; Park, S.; Kim, Y.; Yoon, H. ML-Based 5G Traffic Generation for Practical Simulations Using Open Datasets. IEEE Commun. Mag. 2023, 61, 130–136. https://doi.org/10.1109/MCOM.001.2200679.

Joblib: Running Python Functions as Pipeline Jobs—Joblib 1.5.2 Documentation. Available online: https://joblib.readthedocs.io/en/stable/ (accessed on 22 October 2025).

Wang, C.; Xu, T.; Qin, X. Network Traffic Classification with Improved Random Forest. In Proceedings of the 2015 11th International Conference on Computational Intelligence and Security (CIS), Shenzhen, China, 19–20 December 2015; pp. 78–81. https://doi.org/10.1109/CIS.2015.27.

Scikit-Learn: Machine Learning in Python—Scikit-Learn 1.7.1 Documentation. Available online: https://scikit-learn.org/stable/index.html (accessed on 22 July 2025).

Lemaïtre, G.; Nogueira, F. Imbalanced-learn: A Python Toolbox to Tackle the Curse of Imbalanced Datasets in Machine Learning. J. Mach. Learn. Res. 2017, 18, 1–5.

Breiman, L. Random Forests. Mach. Learn. 2001, 45, 5–32. https://doi.org/10.1023/A:1010933404324.

Pell, R.; Shojafar, M.; Kosmanos, D.; Moschoyiannis, S. Service Classification of Network Traffic in 5G Core Networks using Machine Learning. In 2023 IEEE International Conference on Edge Computing and Communications (EDGE), Chicago, IL, USA; IEEE: Piscataway, NJ, USA, 2023; pp. 309–318. https://doi.org/10.1109/edge60047.2023.00053.

Moore, A.W.; Zuev, D. Internet traffic classification using bayesian analysis techniques. In Proceedings of the 2005 ACM SIGMETRICS International Conference on Measurement and Modeling of Computer Systems; ACM: Banff, AB, Canada, 2005; pp. 50–60. https://doi.org/10.1145/1064212.1064220.

Baker, F.; Babiarz, J.; Chan, K.H. Configuration Guidelines for DiffServ Service Classes. Internet Engineering Task Force. 2006. Available online: https://www.rfc-editor.org/info/rfc4594 (accessed on 29 May 2025).

Wolford, B. What Is GDPR, the EU’s New Data Protection Law? In GDPR.eu. 2018. Available online: https://gdpr.eu/what-is-gdpr/ (accessed on 19 October 2025).

IANA Service Names and Port Numbers. Available online: https://www.iana.org/assignments/service-names-port-numbers/service-names-portnumbers.xhtml (accessed on 15 March 2025).

Wang, X.; Jiang, J.; Tang, Y.; Liu, B.; Wang, X. StriD2FA: Scalable Regular Expression Matching for Deep Packet Inspection. In 2011 IEEE International Conference on Communications (ICC); IEEE: Piscataway, NJ, USA, 2011; pp. 1–5. https://doi.org/10.1109/icc.2011.5963289.

StandardScaler. In Scikit-Learn. Available online: https://scikit-learn/stable/modules/generated/sklearn.preprocessing.StandardScaler.html (accessed on 9 October 2025).

Disclaimer/Publisher’s Note: The statements, opinions and data contained in all publications are solely those of the individual author(s) and contributor(s) and not of MDPI and/or the editor(s). MDPI and/or the editor(s) disclaim responsibility for any injury to people or property resulting from any ideas, methods, instructions or products referred to in the content.